ΠΜΣ «Εκπαίδευση STEM & Συστήματα Εκπαιδευτικών Ρομποτικών Διατάξεων»

Εργαστήριο Εκπαιδευτικής Ρομποτικής Ι (Arduino)

Πηγαίοι κώδικες από το Φύλλο Εργαστηριακής Άσκησης: Ήχος (και ολίγο φως)

Γνωριμία με το buzzer στην ΠΡΑΞΗ

ΠΡΩΤΟ ΒΗΜΑ:

void setup() {

pinMode(8,OUTPUT);

}

void loop() {

digitalWrite(8,HIGH);

delay(200);

digitalWrite(8,LOW);

delay(200);

}

ΤΡΙΤΟ ΒΗΜΑ:

int i;

void setup() {

pinMode(9,OUTPUT);

}

void loop() {

for (i=0;i<=255;i=i+10)

{

analogWrite(9, i);

delay(1000);

}

}

ΠΕΜΠΤΟ ΒΗΜΑ:

int buzzer=8;       //set the digital IO pin to control the buzzer

void setup()

{

pinMode(buzzer,OUTPUT);     //set digital IO pin as output mode

}

void loop()

{

unsigned char i,j;     //Define variable

while(1)

{

for(i=0;i<80;i++)      //Make sound with same frequency

{

digitalWrite(buzzer,HIGH);      //Make sound

delay(1);                                     //Delay 1ms

digitalWrite(buzzer,LOW);     //No sound

delay(1);                                   //Delay 1ms

}

for(i=0;i<100;i++)                   // Make sound with other frequency

{

digitalWrite(buzzer,HIGH);      //Make Sound

delay(2);                                     //Delay2ms

digitalWrite(buzzer,LOW);      //No sounds

delay(2);                                    //Delay 2ms

}

}

}

Σύνδεση με παλιότερες εφαρμογές μας

ΜΙΑ πιθανή λύση

int buzzer=8;

int zzz;

void setup()

{

pinMode(buzzer,OUTPUT);

}

void loop()

{

unsigned char i;

zzz=analogRead(A0);

if (zzz>100)

{

for(i=0;i<100;i++)

{

digitalWrite(buzzer,HIGH);

delay(3);

digitalWrite(buzzer,LOW);

delay(3);

}

for(i=0;i<50;i++)

{

digitalWrite(buzzer,HIGH);

delay(10);

digitalWrite(buzzer,LOW);

delay(10);

}

}

}

ΔΕΥΤΕΡΟ ΜΕΡΟΣ _ ΜΟΥΣΙΚΗ?

ΠΡΩΤΟ ΒΗΜΑ:

#define NOTE_C3  131

#define NOTE_D3  147

#define NOTE_G3  196

#define NOTE_A3  220

#define NOTE_B3  247

#define NOTE_C4  262

#define BUZZER_PIN 8

int melody[] = {

NOTE_C4, NOTE_G3, NOTE_G3, NOTE_A3, NOTE_G3, 0, NOTE_B3, NOTE_C4

};

// note durations: 4 = quarter note, 8 = eighth note, etc.:

int noteDurations[] = {

4, 8, 8, 4, 4, 4, 4, 4

};

void setup() {

// iterate over the notes of the melody:

for (int thisNote = 0; thisNote < 8; thisNote++) {

// to calculate the note duration, take one second divided by the note type.

//e.g. quarter note = 1000 / 4, eighth note = 1000/8, etc.

int noteDuration = 1000 / noteDurations[thisNote];

tone(8, melody[thisNote], noteDuration);

// to distinguish the notes, set a minimum time between them.

// the note's duration + 30% seems to work well:

int pauseBetweenNotes = noteDuration * 1.30;

delay(pauseBetweenNotes);

// stop the tone playing:

noTone(8);

}

}

void loop() {

// no need to repeat the melody.

}

ΔΕΥΤΕΡΟ ΒΗΜΑ: Μια πιθανή λύση

#define NOTE_C3  131

#define NOTE_D3  147

#define NOTE_G3  196

#define NOTE_A3  220

#define NOTE_B3  247

#define NOTE_C4  262

#define BUZZER_PIN 8

int melody[] = {

NOTE_C4, NOTE_G3, NOTE_G3, NOTE_A3, NOTE_G3, 0, NOTE_B3, NOTE_C4

};

// note durations: 4 = quarter note, 8 = eighth note, etc.:

int noteDurations[] = {

4, 8, 8, 4, 4, 4, 4, 4

};

void setup() {

}

void loop() {

// iterate over the notes of the melody:

for (int thisNote = 0; thisNote < 8; thisNote++) {

// to calculate the note duration, take one second divided by the note type.

//e.g. quarter note = 1000 / 4, eighth note = 1000/8, etc.

int noteDuration = 1000 / noteDurations[thisNote];

tone(8, melody[thisNote], noteDuration);

// to distinguish the notes, set a minimum time between them.

// the note's duration + 30% seems to work well:

int pauseBetweenNotes = noteDuration * 1.30;

delay(pauseBetweenNotes);

// stop the tone playing:

noTone(8);

}

delay (2000);

}

ΠΑΡΑΡΤΗΜΑ 1   ΡΟΖ ΠΑΝΘΗΡΑΣ

/*

************************************************

Public Constants

************************************************

*/

#define NOTE_B0  31

#define NOTE_C1  33

#define NOTE_CS1 35

#define NOTE_D1  37

#define NOTE_DS1 39

#define NOTE_E1  41

#define NOTE_F1  44

#define NOTE_FS1 46

#define NOTE_G1  49

#define NOTE_GS1 52

#define NOTE_A1  55

#define NOTE_AS1 58

#define NOTE_B1  62

#define NOTE_C2  65

#define NOTE_CS2 69

#define NOTE_D2  73

#define NOTE_DS2 78

#define NOTE_E2  82

#define NOTE_F2  87

#define NOTE_FS2 93

#define NOTE_G2  98

#define NOTE_GS2 104

#define NOTE_A2  110

#define NOTE_AS2 117

#define NOTE_B2  123

#define NOTE_C3  131

#define NOTE_CS3 139

#define NOTE_D3  147

#define NOTE_DS3 156

#define NOTE_E3  165

#define NOTE_F3  175

#define NOTE_FS3 185

#define NOTE_G3  196

#define NOTE_GS3 208

#define NOTE_A3  220

#define NOTE_AS3 233

#define NOTE_B3  247

#define NOTE_C4  262

#define NOTE_CS4 277

#define NOTE_D4  294

#define NOTE_DS4 311

#define NOTE_E4  330

#define NOTE_F4  349

#define NOTE_FS4 370

#define NOTE_G4  392

#define NOTE_GS4 415

#define NOTE_A4  440

#define NOTE_AS4 466

#define NOTE_B4  494

#define NOTE_C5  523

#define NOTE_CS5 554

#define NOTE_D5  587

#define NOTE_DS5 622

#define NOTE_E5  659

#define NOTE_F5  698

#define NOTE_FS5 740

#define NOTE_G5  784

#define NOTE_GS5 831

#define NOTE_A5  880

#define NOTE_AS5 932

#define NOTE_B5  988

#define NOTE_C6  1047

#define NOTE_CS6 1109

#define NOTE_D6  1175

#define NOTE_DS6 1245

#define NOTE_E6  1319

#define NOTE_F6  1397

#define NOTE_FS6 1480

#define NOTE_G6  1568

#define NOTE_GS6 1661

#define NOTE_A6  1760

#define NOTE_AS6 1865

#define NOTE_B6  1976

#define NOTE_C7  2093

#define NOTE_CS7 2217

#define NOTE_D7  2349

#define NOTE_DS7 2489

#define NOTE_E7  2637

#define NOTE_F7  2794

#define NOTE_FS7 2960

#define NOTE_G7  3136

#define NOTE_GS7 3322

#define NOTE_A7  3520

#define NOTE_AS7 3729

#define NOTE_B7  3951

#define NOTE_C8  4186

#define NOTE_CS8 4435

#define NOTE_D8  4699

#define NOTE_DS8 4978

/*

Melody

Plays a melody

circuit:

- 8 ohm speaker on digital pin 8

*/

// #include "pitches.h"

#define BUZZER_PIN 8

#define REST 0

int melody[] = {

REST, REST, REST, NOTE_DS4,

NOTE_E4, REST, NOTE_FS4, NOTE_G4, REST, NOTE_DS4,

NOTE_E4, NOTE_FS4,  NOTE_G4, NOTE_C5, NOTE_B4, NOTE_E4, NOTE_G4, NOTE_B4,

NOTE_AS4, NOTE_A4, NOTE_G4, NOTE_E4, NOTE_D4,

NOTE_E4, REST, REST, NOTE_DS4,

NOTE_E4, REST, NOTE_FS4, NOTE_G4, REST, NOTE_DS4,

NOTE_E4, NOTE_FS4,  NOTE_G4, NOTE_C5, NOTE_B4, NOTE_G4, NOTE_B4, NOTE_E5,

NOTE_DS5,

NOTE_D5, REST, REST, NOTE_DS4,

NOTE_E4, REST, NOTE_FS4, NOTE_G4, REST, NOTE_DS4,

NOTE_E4, NOTE_FS4,  NOTE_G4, NOTE_C5, NOTE_B4, NOTE_E4, NOTE_G4, NOTE_B4,

NOTE_AS4, NOTE_A4, NOTE_G4, NOTE_E4, NOTE_D4,

NOTE_E4, REST,

REST, NOTE_E5, NOTE_D5, NOTE_B4, NOTE_A4, NOTE_G4, NOTE_E4,

NOTE_AS4, NOTE_A4, NOTE_AS4, NOTE_A4, NOTE_AS4, NOTE_A4, NOTE_AS4, NOTE_A4,

NOTE_G4, NOTE_E4, NOTE_D4, NOTE_E4, NOTE_E4, NOTE_E4

};

int durations[] = {

2, 4, 8, 8,

4, 8, 8, 4, 8, 8,

8, 8,  8, 8, 8, 8, 8, 8,

2, 16, 16, 16, 16,

2, 4, 8, 4,

4, 8, 8, 4, 8, 8,

8, 8,  8, 8, 8, 8, 8, 8,

1,

2, 4, 8, 8,

4, 8, 8, 4, 8, 8,

8, 8,  8, 8, 8, 8, 8, 8,

2, 16, 16, 16, 16,

4, 4,

4, 8, 8, 8, 8, 8, 8,

16, 8, 16, 8, 16, 8, 16, 8,

16, 16, 16, 16, 16, 2

};

void setup()

{

pinMode(BUZZER_PIN, OUTPUT);

}

void loop()

{

int size = sizeof(durations) / sizeof(int);

for (int note = 0; note < size; note++) {

//to calculate the note duration, take one second divided by the note type.

//e.g. quarter note = 1000 / 4, eighth note = 1000/8, etc.

int duration = 1000 / durations[note];

tone(BUZZER_PIN, melody[note], duration);

//to distinguish the notes, set a minimum time between them.

//the note's duration + 30% seems to work well:

int pauseBetweenNotes = duration * 1.30;

delay(pauseBetweenNotes);

//stop the tone playing:

noTone(BUZZER_PIN);

}

}